Secure Hotel WLAN Network

Complete Device Configuration Reference v2.0

Version 2.0 | April 2026 | Updated with SSH, NTP, Syslog, QoS

# 1. ISP Routers

## 1.1 ISP-ROUTER-1

Model: ISR4331 | Role: ISP Router 1 (Primary) | Added: SSH, NTP, Syslog

hostname ISP-ROUTER-1

!

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

no ip domain-lookup

!

interface GigabitEthernet0/0/0

description TO-CLOUD

ip address 200.100.50.10 255.255.255.252

!

interface GigabitEthernet0/0/1

description TO-OUTSIDE-SW1

ip address 200.100.50.21 255.255.255.252

!

interface GigabitEthernet0/0/2

description TO-OUTSIDE-SW2

ip address 200.100.50.25 255.255.255.252

!

ip route 0.0.0.0 0.0.0.0 200.100.50.9

ip route 192.168.10.0 255.255.255.0 200.100.50.9

ip route 200.100.50.44 255.255.255.252 200.100.50.9

ip route 200.100.50.48 255.255.255.252 200.100.50.9

ip route 200.100.50.52 255.255.255.252 200.100.50.9

ip route 200.100.50.56 255.255.255.252 200.100.50.9

ip route 200.100.50.60 255.255.255.252 200.100.50.9

ip route 200.100.50.64 255.255.255.252 200.100.50.9

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

!

! === NTP ===

ntp server 10.10.1.228

ntp update-calendar

clock timezone ICT 7

!

! === Syslog ===

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

## 1.2 ISP-ROUTER-2 และ ISP-ROUTER-3

Config เหมือน ISP-ROUTER-1 แต่เปลี่ยน IP ตามตารางด้านล่าง:

| Router | TO-CLOUD | TO-SW1 | TO-SW2 |
| --- | --- | --- | --- |
| ISP-R2 | 200.100.50.14/30 | 200.100.50.29/30 | 200.100.50.33/30 |
| ISP-R3 | 200.100.50.18/30 | 200.100.50.37/30 | 200.100.50.41/30 |

# 2. Outside Switches (DMZ Outside)

## 2.1 OUTSIDE-SW1

Model: 3650-24PS | Added: SSH, NTP, Syslog

hostname OUTSIDE-SW1

ip routing

no ip domain-lookup

!

interface GigabitEthernet1/0/1

description TO-ISP-ROUTER-1

no switchport

ip address 200.100.50.22 255.255.255.252

!

interface GigabitEthernet1/0/2

description TO-ISP-ROUTER-2

no switchport

ip address 200.100.50.30 255.255.255.252

!

interface GigabitEthernet1/0/3

description TO-ISP-ROUTER-3

no switchport

ip address 200.100.50.38 255.255.255.252

!

interface GigabitEthernet1/0/4

description TO-PERIMETER-FIREWALL-1

no switchport

ip address 200.100.50.45 255.255.255.252

!

interface GigabitEthernet1/0/5

description TO-PERIMETER-FIREWALL-2

no switchport

ip address 200.100.50.49 255.255.255.252

!

interface GigabitEthernet1/0/6

description TO-PERIMETER-FIREWALL-3

no switchport

ip address 200.100.50.53 255.255.255.252

!

interface GigabitEthernet1/0/7

description TO-OUTSIDE-SW2-LINK1

no switchport

ip address 200.100.50.69 255.255.255.252

!

interface GigabitEthernet1/0/8

description TO-OUTSIDE-SW2-LINK2

no switchport

ip address 200.100.50.73 255.255.255.252

!

interface GigabitEthernet1/0/9

description TO-INTERNET-ROUTER

no switchport

ip address 200.100.50.78 255.255.255.252

!

ip route 0.0.0.0 0.0.0.0 200.100.50.21 1

ip route 0.0.0.0 0.0.0.0 200.100.50.29 2

ip route 0.0.0.0 0.0.0.0 200.100.50.37 3

ip route 192.168.10.0 255.255.255.0 200.100.50.77

ip route 200.100.50.44 255.255.255.252 200.100.50.10 1

ip route 200.100.50.48 255.255.255.252 200.100.50.10 1

ip route 200.100.50.52 255.255.255.252 200.100.50.10 1

ip route 200.100.50.56 255.255.255.252 200.100.50.10 1

ip route 200.100.50.60 255.255.255.252 200.100.50.10 1

ip route 200.100.50.64 255.255.255.252 200.100.50.10 1

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

ntp update-calendar

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

OUTSIDE-SW2: Config เหมือนกัน เปลี่ยน IP ตามที่กำหนด (SW2: .26/.34/.42/.57/.61/.65/.70/.74)

# 3. Perimeter Firewalls

## 3.1 PERIMETER-FIREWALL-1

Model: ASA 5506-X | Added: NAT, ACL, ICMP Inspection

ASA Version 9.6(1)

hostname PERIMETER-FIREWALL-1

!

interface GigabitEthernet1/1

description TO-OUTSIDE-SW1

nameif outside1

security-level 0

ip address 200.100.50.46 255.255.255.252

!

interface GigabitEthernet1/2

description TO-OUTSIDE-SW2

nameif outside2

security-level 0

ip address 200.100.50.58 255.255.255.252

!

interface GigabitEthernet1/3

description TO-INSIDE-SW1

nameif inside1

security-level 100

ip address 10.10.4.1 255.255.255.252

!

interface GigabitEthernet1/4

description TO-INSIDE-SW2

nameif inside2

security-level 100

ip address 10.10.4.5 255.255.255.252

!

! === NAT ===

object network INTERNAL-GUEST

subnet 10.10.0.0 255.255.0.0

nat (inside1,outside1) dynamic interface

!

object network INTERNAL-VOIP

subnet 172.16.0.0 255.255.0.0

nat (inside1,outside1) dynamic interface

!

! === ACL ===

access-list INSIDE_OUT extended permit ip any any

access-group INSIDE_OUT in interface inside1

access-group INSIDE_OUT in interface inside2

access-list OUTSIDE_IN extended permit ip any any

access-group OUTSIDE_IN in interface outside1

access-group OUTSIDE_IN in interface outside2

!

! === Routing ===

route inside1 10.10.0.0 255.255.0.0 10.10.4.2 1

route inside1 172.16.0.0 255.255.0.0 10.10.4.2 1

route inside1 10.10.4.0 255.255.252.0 10.10.4.2 1

route outside1 0.0.0.0 0.0.0.0 200.100.50.45 1

route outside2 0.0.0.0 0.0.0.0 200.100.50.57 2

!

! === Inspection Policy ===

class-map inspection_default

match default-inspection-traffic

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect tftp

inspect icmp

service-policy global_policy global

!

end

FW-2: เปลี่ยน IP outside1=200.100.50.50, outside2=200.100.50.62, inside1=10.10.4.9, inside2=10.10.4.13

FW-3: เปลี่ยน IP outside1=200.100.50.54, outside2=200.100.50.66, inside1=10.10.4.17, inside2=10.10.4.21

# 4. Inside Switches (DMZ Inside)

## 4.1 INSIDE-SW1

Model: 3650-24PS | Added: OSPF, SSH, NTP, Syslog

hostname INSIDE-SW1

ip routing

no ip domain-lookup

!

interface GigabitEthernet0/1

description TO-PERIMETER-FIREWALL-1

no switchport

ip address 10.10.4.2 255.255.255.252

!

interface GigabitEthernet0/2

description TO-PERIMETER-FIREWALL-2

no switchport

ip address 10.10.4.10 255.255.255.252

!

interface FastEthernet0/1

description TO-PERIMETER-FIREWALL-3

no switchport

ip address 10.10.4.18 255.255.255.252

!

interface FastEthernet0/2

description TO-COREROUTER1

no switchport

ip address 10.10.4.33 255.255.255.252

!

interface FastEthernet0/3

description TO-COREROUTER2

no switchport

ip address 10.10.4.37 255.255.255.252

!

interface FastEthernet0/4

description TO-INSIDE-SW2-LINK1

no switchport

ip address 10.10.4.25 255.255.255.252

!

interface FastEthernet0/5

description TO-INSIDE-SW2-LINK2

no switchport

ip address 10.10.4.29 255.255.255.252

!

router ospf 1

router-id 3.3.3.3

network 10.10.4.0 0.0.0.3 area 0

network 10.10.4.8 0.0.0.3 area 0

network 10.10.4.16 0.0.0.3 area 0

network 10.10.4.32 0.0.0.3 area 0

network 10.10.4.36 0.0.0.3 area 0

network 10.10.4.24 0.0.0.3 area 0

network 10.10.4.28 0.0.0.3 area 0

default-information originate

!

ip route 0.0.0.0 0.0.0.0 10.10.4.1 1

ip route 0.0.0.0 0.0.0.0 10.10.4.9 2

ip route 0.0.0.0 0.0.0.0 10.10.4.17 3

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

ntp update-calendar

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

INSIDE-SW2: router-id=4.4.4.4, IP: Gi0/1=10.10.4.6, Gi0/2=10.10.4.14, Fa0/1=10.10.4.22, Fa0/2=10.10.4.41, Fa0/3=10.10.4.45, default route via 10.10.4.5/13/21

# 5. Core Routers

## 5.1 Hotel-Core-Router1

Model: Cisco 2811 + NM-2FE2W | Added: SSH, NTP, Syslog

hostname Hotel-Core-Router1

no ip domain-lookup

no ip cef

!

interface FastEthernet0/0

description TO-INSIDE-SW1

ip address 10.10.4.34 255.255.255.252

!

interface FastEthernet0/1

description TO-INSIDE-SW2

ip address 10.10.4.42 255.255.255.252

!

interface FastEthernet1/0

description TO-BLOCK-A-MLSW1

ip address 10.10.4.49 255.255.255.252

!

interface FastEthernet1/1

description TO-BLOCK-A-MLSW2

ip address 10.10.4.53 255.255.255.252

!

router ospf 1

router-id 1.1.1.1

network 10.10.4.32 0.0.0.3 area 0

network 10.10.4.40 0.0.0.3 area 0

network 10.10.4.48 0.0.0.3 area 0

network 10.10.4.52 0.0.0.3 area 0

default-information originate

!

ip route 0.0.0.0 0.0.0.0 10.10.4.33 1

ip route 0.0.0.0 0.0.0.0 10.10.4.41 2

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

ntp update-calendar

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

Hotel-Core-Router2: router-id=2.2.2.2, Fa0/0=10.10.4.38, Fa0/1=10.10.4.46, Fa1/0=10.10.4.57, Fa1/1=10.10.4.61, default route via 10.10.4.37/10.10.4.45

# 6. Distribution Layer (BLOCK-A-MLSW)

## 6.1 BLOCK-A-MLSW1

Model: 3650-24PS | HSRP Active | LACP Desirable | Added: SSH, NTP, Syslog, QoS

hostname BLOCK-A-MLSW1

ip routing

no ip domain-lookup

spanning-tree mode pvst

!

! === LACP EtherChannel ===

interface Port-channel1

description TO-MLSW2-LACP

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,55,60,99,199

!

interface GigabitEthernet1/0/1

description TO-COREROUTER1

no switchport

ip address 10.10.4.50 255.255.255.252

!

interface GigabitEthernet1/0/2

description TO-COREROUTER2

no switchport

ip address 10.10.4.58 255.255.255.252

!

interface GigabitEthernet1/0/3

description TO-MLSW2-LINK1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,55,60,99,199

channel-group 1 mode desirable

!

interface GigabitEthernet1/0/4

description TO-MLSW2-LINK2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,55,60,99,199

channel-group 1 mode desirable

!

interface GigabitEthernet1/0/5

description TO-DC-SWITCH

switchport mode trunk

switchport trunk allowed vlan 99,199

!

interface GigabitEthernet1/0/6

description TO-GROUND-FLOOR-SW

switchport mode trunk

switchport trunk allowed vlan 60,99

!

interface GigabitEthernet1/0/7

description TO-F1-RM1-130

switchport mode trunk

switchport trunk allowed vlan 10,99

!

interface GigabitEthernet1/0/8

description TO-F1-RM11-60

switchport mode trunk

switchport trunk allowed vlan 10,99

!

interface GigabitEthernet1/0/9

description TO-F2-RM61-90

switchport mode trunk

switchport trunk allowed vlan 20,99

!

interface GigabitEthernet1/0/10

description TO-F2-RM91-120

switchport mode trunk

switchport trunk allowed vlan 20,99

!

interface GigabitEthernet1/0/11

description TO-F3-RM121-150

switchport mode trunk

switchport trunk allowed vlan 30,99

!

interface GigabitEthernet1/0/12

description TO-F3-RM151-180

switchport mode trunk

switchport trunk allowed vlan 30,99

!

interface GigabitEthernet1/0/13

description TO-F4-RM181-210

switchport mode trunk

switchport trunk allowed vlan 40,99

!

interface GigabitEthernet1/0/14

description TO-F4-RM211-240

switchport mode trunk

switchport trunk allowed vlan 40,99

!

interface GigabitEthernet1/0/15

description TO-F5-RM241-270

switchport mode trunk

switchport trunk allowed vlan 50,99

!

interface GigabitEthernet1/0/16

description TO-F5-RM271-300

switchport mode trunk

switchport trunk allowed vlan 50,99

!

interface GigabitEthernet1/0/17

description TO-EMP-SWITCH

switchport mode trunk

switchport trunk allowed vlan 55,99

!

! === SVI + HSRP (Active) + DHCP Relay ===

interface Vlan10

ip address 10.10.28.2 255.255.254.0

ip helper-address 10.10.1.228

standby 10 ip 10.10.28.1

standby 10 priority 110

standby 10 preempt

ip access-group GUEST-ACL in

!

interface Vlan20

ip address 10.10.30.2 255.255.254.0

ip helper-address 10.10.1.228

standby 20 ip 10.10.30.1

standby 20 priority 110

standby 20 preempt

ip access-group GUEST-ACL in

!

interface Vlan30

ip address 10.10.32.2 255.255.254.0

ip helper-address 10.10.1.228

standby 30 ip 10.10.32.1

standby 30 priority 110

standby 30 preempt

ip access-group GUEST-ACL in

!

interface Vlan40

ip address 10.10.34.2 255.255.254.0

ip helper-address 10.10.1.228

standby 40 ip 10.10.34.1

standby 40 priority 110

standby 40 preempt

ip access-group GUEST-ACL in

!

interface Vlan50

ip address 10.10.36.2 255.255.254.0

ip helper-address 10.10.1.228

standby 50 ip 10.10.36.1

standby 50 priority 110

standby 50 preempt

ip access-group GUEST-ACL in

!

interface Vlan55

ip address 10.10.20.2 255.255.254.0

ip helper-address 10.10.1.228

standby 55 ip 10.10.20.1

standby 55 priority 110

standby 55 preempt

!

interface Vlan60

ip address 10.10.38.2 255.255.254.0

ip helper-address 10.10.1.228

standby 60 ip 10.10.38.1

standby 60 priority 110

standby 60 preempt

ip access-group GUEST-ACL in

!

interface Vlan99

ip address 172.16.10.2 255.255.0.0

ip helper-address 10.10.1.228

standby 99 ip 172.16.10.1

standby 99 priority 110

standby 99 preempt

!

interface Vlan199

ip address 10.10.1.226 255.255.255.224

standby 199 ip 10.10.1.225

standby 199 priority 110

standby 199 preempt

!

! === ACL ===

ip access-list extended GUEST-ACL

permit udp any host 10.10.1.228 eq bootps

permit udp any host 10.10.1.229 eq domain

permit tcp any host 10.10.1.231 eq www

permit tcp any host 10.10.1.231 eq 443

permit tcp any host 10.10.1.230 eq smtp

permit tcp any host 10.10.1.230 eq pop3

deny ip 10.10.28.0 0.0.3.255 10.10.20.0 0.0.1.255

deny ip 10.10.28.0 0.0.3.255 10.10.1.224 0.0.0.31

permit ip any any

!

! === QoS ===

class-map match-any VOICE-TRAFFIC

match ip dscp ef

!

class-map match-any VOIP-SIGNALING

match ip dscp cs3

!

policy-map QOS-POLICY

class VOICE-TRAFFIC

priority percent 30

class VOIP-SIGNALING

bandwidth percent 5

class class-default

fair-queue

!

interface GigabitEthernet1/0/1

service-policy output QOS-POLICY

interface GigabitEthernet1/0/2

service-policy output QOS-POLICY

!

! === OSPF ===

router ospf 1

router-id 5.5.5.5

network 10.10.4.48 0.0.0.3 area 0

network 10.10.4.56 0.0.0.3 area 0

network 10.10.28.0 0.0.1.255 area 0

network 10.10.30.0 0.0.1.255 area 0

network 10.10.32.0 0.0.1.255 area 0

network 10.10.34.0 0.0.1.255 area 0

network 10.10.36.0 0.0.1.255 area 0

network 10.10.38.0 0.0.1.255 area 0

network 10.10.20.0 0.0.1.255 area 0

network 172.16.10.0 0.0.255.255 area 0

network 10.10.1.224 0.0.0.31 area 0

!

ip route 0.0.0.0 0.0.0.0 10.10.4.49 1

ip route 0.0.0.0 0.0.0.0 10.10.4.57 2

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

ntp update-calendar

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

BLOCK-A-MLSW2: router-id=6.6.6.6, IP SVI .3 แทน .2, HSRP priority=90, channel-group mode auto, default route via 10.10.4.53/10.10.4.61

# 7. Server Farm (DC-SWITCH)

## 7.1 DC-SWITCH

Model: 2960-24TT | Added: SSH, NTP, Syslog, Port Security (WLC port ใช้ restrict)

hostname DC-SWITCH

no ip domain-lookup

spanning-tree mode pvst

!

vlan 99

name VOIP

vlan 199

name SERVER-FARM

!

interface FastEthernet0/1

description EMAIL-SERVER

switchport mode access

switchport access vlan 199

spanning-tree portfast

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface FastEthernet0/2

description DNS-SERVER

switchport mode access

switchport access vlan 199

spanning-tree portfast

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface FastEthernet0/3

description HTTP-WEB-SERVER

switchport mode access

switchport access vlan 199

spanning-tree portfast

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface FastEthernet0/4

description DHCP-SERVER

switchport mode access

switchport access vlan 199

spanning-tree portfast

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface FastEthernet0/5

description NET-ADMIN-PC

switchport mode access

switchport access vlan 199

spanning-tree portfast

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface FastEthernet0/6

description WLC-LAN-CONTROLLER

switchport mode access

switchport access vlan 199

spanning-tree portfast

! Note: Port Security ปิดเพราะ WLC restart บ่อย MAC เปลี่ยน

!

interface FastEthernet0/7

description VOICE-GATEWAY

switchport mode access

switchport access vlan 99

spanning-tree portfast

mls qos trust dscp

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

!

interface GigabitEthernet0/1

description TO-MLSW1

switchport mode trunk

switchport trunk allowed vlan 99,199

!

interface GigabitEthernet0/2

description TO-MLSW2

switchport mode trunk

switchport trunk allowed vlan 99,199

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

# 8. Access Switches (Guest Block)

## 8.1 Ground Floor Switch

Model: 2960-24TT | VLAN: 60 | Added: Voice VLAN, QoS, SSH, NTP, Syslog

hostname GROUND-FLOOR-SWITCH

no ip domain-lookup

spanning-tree mode pvst

!

vlan 60

name GUEST-GF

vlan 99

name VOIP

!

! === QoS Global ===

mls qos

!

! === VoIP Ports (Voice VLAN + QoS) ===

interface range FastEthernet0/1-4

description VOIP-PHONES

switchport mode access

switchport access vlan 60

switchport voice vlan 99

spanning-tree portfast

mls qos trust dscp

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

! === PC Ports ===

interface range FastEthernet0/5-8

description PC-PORTS

switchport mode access

switchport access vlan 60

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

! === AP Ports (Trunk) ===

interface range FastEthernet0/9-11

description ACCESS-POINTS

switchport mode trunk

switchport trunk allowed vlan 60,99

!

! === Uplinks to MLSW ===

interface GigabitEthernet0/1

description TO-MLSW1

switchport mode trunk

switchport trunk allowed vlan 60,99

spanning-tree portfast trunk

!

interface GigabitEthernet0/2

description TO-MLSW2

switchport mode trunk

switchport trunk allowed vlan 60,99

spanning-tree portfast trunk

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

## 8.2 Floor 1-5 Access Switches

Config เหมือน Ground Floor แต่เปลี่ยนตามตารางด้านล่าง:

| Floor | SW1 | SW2 | VLAN | VoIP Port |
| --- | --- | --- | --- | --- |
| Floor 1 | RM1-130 | RM11-60 | 10 | SW1:Fa0/2 / SW2:Fa0/3 |
| Floor 2 | RM61-90 | RM91-120 | 20 | SW1:Fa0/2 / SW2:Fa0/3 |
| Floor 3 | RM121-150 | RM151-180 | 30 | SW1:Fa0/2 / SW2:Fa0/3 |
| Floor 4 | RM181-210 | RM211-240 | 40 | SW1:Fa0/2 / SW2:Fa0/3 |
| Floor 5 | RM241-270 | RM271-300 | 50 | SW1:Fa0/2 / SW2:Fa0/3 |

# 9. Employee Block Switch

## 9.1 EMP Switch

Model: 2960-24TT | VLAN: 55 | Added: Voice VLAN, QoS, SSH, NTP, Syslog

hostname EMP

no ip domain-lookup

spanning-tree mode pvst

!

vlan 55

name EMPLOYEE

vlan 99

name VOIP

!

mls qos

!

interface range FastEthernet0/1-7

description PC-PORTS

switchport mode access

switchport access vlan 55

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

interface range FastEthernet0/8-14

description VOIP-PHONES

switchport mode access

switchport access vlan 55

switchport voice vlan 99

spanning-tree portfast

mls qos trust dscp

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

interface range FastEthernet0/15-18

description PRINTER-PORTS

switchport mode access

switchport access vlan 55

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

interface range FastEthernet0/19-22

description LAPTOP-PORTS

switchport mode access

switchport access vlan 55

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

interface GigabitEthernet0/1

description TO-MLSW1

switchport mode trunk

switchport trunk allowed vlan 55,99

spanning-tree portfast trunk

!

interface GigabitEthernet0/2

description TO-MLSW2

switchport mode trunk

switchport trunk allowed vlan 55,99

spanning-tree portfast trunk

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 10.10.1.228

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

# 10. Voice Gateway (VoIP CME)

## 10.1 VOICE-GATEWAY

Model: Cisco 2811 | Added: SSH, NTP, Syslog

hostname VOICE-GATEWAY

no ip domain-lookup

!

interface FastEthernet0/0

description TO-DC-SWITCH

ip address 172.16.10.10 255.255.0.0

no shutdown

!

ip route 0.0.0.0 0.0.0.0 172.16.10.1

!

telephony-service

max-ephones 30

max-dn 30

ip source-address 172.16.10.10 port 2000

auto assign 1 to 30

keepalive 120

!

ephone-dn 1

number 1001

ephone-dn 2

number 1002

ephone-dn 3

number 1003

ephone-dn 4

number 1004

ephone-dn 5

number 1005

ephone-dn 6

number 1006

ephone-dn 7

number 1007

ephone-dn 8

number 1008

ephone-dn 9

number 1009

ephone-dn 10

number 1010

ephone-dn 11

number 1011

ephone-dn 12

number 1012

ephone-dn 13

number 1013

ephone-dn 14

number 1014

ephone-dn 15

number 1015

ephone-dn 16

number 1016

ephone-dn 17

number 1017

ephone-dn 18

number 1018

ephone-dn 19

number 1019

ephone-dn 20

number 1020

!

! === Security Hardening ===

service password-encryption

enable secret Cisco@123

username admin privilege 15 secret Cisco@123

ip domain-name hotel.com

crypto key generate rsa

! --> 1024 bits

ip ssh version 2

ip ssh time-out 60

ip ssh authentication-retries 3

ntp server 172.16.10.1

ntp update-calendar

clock timezone ICT 7

logging host 10.10.1.228

service timestamps log datetime msec

!

line vty 0 4

transport input ssh

login local

exec-timeout 10 0

line con 0

password Cisco@123

login

exec-timeout 10 0

!

end

# 11. Internet Router

## 11.1 INTERNET-ROUTER1

Model: Cisco 2811 + NM-2FE2W | Role: Simulate Internet User

hostname INTERNET-ROUTER1

no ip domain-lookup

!

interface FastEthernet0/0

description TO-TEST-PC

ip address 192.168.10.254 255.255.255.0

no shutdown

!

interface FastEthernet0/1

description TO-OUTSIDE-SW

ip address 200.100.50.77 255.255.255.252

no shutdown

!

ip route 0.0.0.0 0.0.0.0 200.100.50.78

ip route 200.100.50.44 255.255.255.252 200.100.50.78

ip route 200.100.50.48 255.255.255.252 200.100.50.78

ip route 200.100.50.52 255.255.255.252 200.100.50.78

ip route 200.100.50.56 255.255.255.252 200.100.50.78

ip route 200.100.50.60 255.255.255.252 200.100.50.78

ip route 200.100.50.64 255.255.255.252 200.100.50.78

!

end

# 12. Server & WLC Configuration

## 12.1 Server IP Configuration (Static)

| Device | IP | Subnet | Gateway | DNS |
| --- | --- | --- | --- | --- |
| DHCP Server | 10.10.1.228 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| DNS Server | 10.10.1.229 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| EMAIL Server | 10.10.1.230 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| HTTP/WEB Server | 10.10.1.231 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| WLC-2504 | 10.10.1.232 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| NET Admin-PC | 10.10.1.233 | 255.255.255.224 | 10.10.1.225 | 10.10.1.229 |
| VOICE GW | 172.16.10.10 | 255.255.0.0 | 172.16.10.1 | 10.10.1.229 |
| TEST-PC | 192.168.10.1 | 255.255.255.0 | 192.168.10.254 | - |

## 12.2 WLC-2504 Configuration

| Parameter | Value |
| --- | --- |
| System Name | Hotel-WLC |
| Management IP | 10.10.1.232 |
| Management VLAN | 199 |
| Default Gateway | 10.10.1.225 |
| SSID (Employee) | Hotel-Employee |
| SSID (Guest) | Hotel-Guest |
| Security | WPA2-Personal, AES |
| Password | Cisco@123 |
| Web Login | admin / Cisco@123 |

## 12.3 AP Configuration (Autonomous AP)

ตั้งค่าที่ AP ธรรมดาทุกตัว (18 ตัว) ผ่าน Config > Port 1:

| Parameter | Value |
| --- | --- |
| SSID | Hotel-Guest |
| Authentication | WPA2-PSK |
| PSK Pass Phrase | Cisco@123 |

LAP-PT (Lightweight AP) ในส่วน Employee Block รับ config จาก WLC อัตโนมัติ ไม่ต้องตั้งค่าเอง

--- End of Configuration Document v2.0 ---